UHF rfid integrated reader

OPP-D03

1.The synopsis of the reader：

The OPP-D03 is a low profile, high-performance UHF rfid integrated reader. Adopt Impinj IndyR2000 chip, with high-performance multi-tag identification algorithms.

The OPP-D03 has built-in Linux operating system, which not only meets the complex customer business requirements, but also responds to various label operations independently and quickly. Multi-tag reading performance is excellent, supporting intensive reading, large capacity tag reading and writing, received signal strength detection (RSSI). It has reliable network adaptability, supports a variety of network protocols, and is easy to deploy large-scale applications.

2.Product features

| Mechanical electrical performance |  |  |
| --- | --- | --- |
| Equipment size Dimension：（MM） |  | 500（L）*700（W）*35（H）（±1） |
| Net weight： (KG) |  | 6.780（±0.02） |
| Shell |  | Black acrylic panel, milky white metal bottom shell, the whole machine is black |
| Antenna |  | integrated 4pcs 8 dBiC antennas |
| Performance Index |  |  |
| Tag / Transponder Protocols |  | EPCglobal Gen 2 (ISO 18000-6C)、ISO 18000-6B (optional) |
| Frequency |  | 865~868MHz/902～928MHz（Can be adjusted according to different countries or regions） |
| RF output |  | from 0 dBm to 30 dBm* with +/-1dBm |
| Reading distance |  | - ＞8m（eg.9662，depends on tag） |
| Peak inventory speed |  | > 700 tags/sec |
| Tag RSSI |  | Supported |
| Antenna Detector |  | Supported |
| Power |  | With 220V AC input, the output of +12 V/5A DC power converter |
| Communication interface |  | RJ45(10/100 Base-T Ethernet)、RS232、USB (for debugging) |
| Operating System |  | Linux |
| Communication protocol |  | RS-232、TCP/IP、 Modbus、HTTP/HTTPS、MQTT、RabbitMQ |
| Host API |  | C#、JAVA |
| Development Environment |  | Linux、Android、Windows、PLC、Web |
| Environment |  |  |
| Operating Temp | -20～+55℃ |  |
| Storage Temp | -20～+85℃ |  |
| Humidity | 5%～95%RH，Non-condensing |  |

- Interface diagram：

Figure 2

Figure 3