# UHF rfid integrated reader

# OPP-D02

- 1.

- OPP-D02 is a high-performance UHF RFID desktop integrated reader, Based on the design of high-performance UHF chip,it fully supports the EPCglobal C1 Gen 2 ISO 18000-6c standard.Combined with the proprietary high-efficiency collision processing algorithm, while maintaining a high reading rate.

- OPP-D02 is suitable for RFID solutions such as logistics,warehousing,express delivery,store management,and financial management.

2.Product parameters：

| Mechanical electrical performance |  |
| --- | --- |
| Dimension:（MM） | 320（L）*225（W）*17（H）（±1） |
| Diemension including packaging（MM） | 370（L）*270（W）*55（H）（±3） |
| Net weight:(KG) | 0.94（±0.1） |
| Gross weight:(KG） | 1.23（±0.1） |
| Shell | Acrylic panel, metal bottom shell, the whole reader is black |
| Performance Index |  |
| RFID Protocol Support | EPCglobal Gen 2 (ISO 18000-6C) |
| RF Transceiver | Default: MagicRF M100, IMPINJ E310 Optional |
| RF Antenan | Built-in a circularly polarized antenna，The gain of the antenna ＞ 0dBi |
| Frequency | 865~868MHz/902～928MHz（Can be adjusted according to different countries or regions） |
| Receiver Sensitivity | MagicRF M100:-69dBm |
|  | IMPINJ E310:-74dBm |
| RF Power Output | MagicRF M100:from 0 dBm to 26dBm(±1 adjustable) |
|  | IMPINJ E310:from 5 dBm to 27dBm(±1 adjustable) |
| Reading distance | - MagicRF M100:＞0.8m（eg.9662，depends on tag） |
|  | - IMPINJ E310:＞2m（eg.9662，depends on tag） |
| Peak inventory speed | MagicRF M100:>50 tags/sec |
|  | MPINJ E310:>200 tags/sec |
| Tag RSSI | Supported |
| Antenna Detector | Supported |
| Indicator light | PWR:Red LED,Power indicator Data:Green LED,Successfully reading the tags will light up green |
| Power | 5V,500mA（MAX） |
| Communication interface | USB type C |
| Certification | CE、ROHS |
| API support | C#、JAVA |
| Environment |  |
| Operating Temp | -20～+55℃ |
| Storage Temp | -20～+85℃ |
| Humidity | 5%～95%RH，Non-condensing |

- 3.The product image:

- Figure 2                                 Figure 3

Figure 4                                   Figure 5

- Directional diagram of the built-in antenna:

Figure 6                                          Figure 7

5.MECHANICAL DATA (UNIT:mm)

Figure 8

- 6.Packing List

| Name | Model/Description | Number |
| --- | --- | --- |
| Reader | OPP-D02 | 1 |
| USB Cable | USB To Type C,The length is 2 meters, with a magnetic ring and supports screw fixation | 1 |
| Package | Paper box+pearl cotton, as shown in the following figure (Figure 9) | 1 |

Figure 9